UART Master-Slave Communication in Verilog

Overview

This Verilog design implements a full-duplex UART Master that supports both serial transmission (TX) and reception (RX) using two independent finite state machines (FSMs). It conforms to standard UART framing: 1 start bit (logic low), 8 data bits (LSB first), and 1 stop bit (logic high). The module utilizes a parameterizable baud rate defined by CLKS_PER_BIT and operates at a system clock frequency of 100 MHz, as configured in the testbench.

Key Features

- Transmitter and Receiver FSMs: Two separate FSMs manage the sending and receiving of UART data frames.

- Configurable Baud Rate: Bit timing is based on the CLKS_PER_BIT parameter, allowing compatibility with different baud rates depending on the system clock.

- FSM Control Logic:

  - TX FSM transitions through IDLE → START_BIT → DATA_BITS → STOP_BIT.

  - RX FSM transitions through IDLE → RX_START_BIT → RX_DATA_BITS → RX_STOP_BIT → CLEANUP.

- Mid-Bit Sampling: The receiver samples each bit at the middle of its period to ensure data stability.

- Handshaking Signals:

  - o_TX_Done pulses high when a byte transmission completes.

  - o_RX_DV pulses high when a byte is successfully received.

Signal Behavior

This module uses a 100 MHz system clock (i_Clock) to manage all timing. To initiate a transmission, the i_TX_DV signal is set high with the byte on i_TX_Byte. The serialized output appears on o_TX_Serial, and o_TX_Done indicates when transmission is finished. The input line i_RX_Serial is continuously monitored. When a start bit is detected, the receiver samples each bit in the middle of its expected time window. Once a valid byte is received, it is placed on o_RX_Byte and o_RX_DV is raised for one clock cycle. The CLKS_PER_BIT parameter defines the number of clock cycles per UART bit (e.g., 868 for 115200 baud at 100 MHz).

Working

The transmitter FSM waits for a valid request, sends a start bit (0), shifts out the 8 bits of the data byte from LSB to MSB, and ends with a stop bit (1). Each bit is held on the serial line for CLKS_PER_BIT clock cycles. The receiver FSM looks for a falling edge indicating a start bit, waits CLKS_PER_BIT/2 to align mid-bit, and then samples each data bit and the stop bit sequentially. When all bits are received correctly, the byte is presented on the output along with a data-valid pulse.

Testbench

The testbench simulates UART operation using a 100 MHz clock (10 ns period). It initiates a transmission of the byte 0x3F, observes the output on the o_TX_Serial line, and checks the internal signal behavior over a sufficient simulation window. A waveform file dump.vcd is generated, which can be visualized in GTKWave to inspect signal transitions, FSM activity, and protocol correctness.

Usage

This UART module is ideal for FPGA or ASIC designs that require UART-based serial communication. It can be used for interfacing with terminals, MCUs, sensors, or other digital systems. With FSM-based modularity, clock configurability, and standard protocol framing, the design is portable, scalable, and robust across a variety of embedded applications.